# MATERIAL SAFETY DATA SHEET

## Section 1 – Product Identification

High Caliper Growing – Smart Pot – Polypropylene Fabric

Manufacturer’s Name:  	High Caliper Growing

115 NW 44th Street

Oklahoma City, OK 73118

Date Prepared:		June 24, 2008

Section 2 – Hazardous Identification

No hazardous components in geotextile fabrics at or above threshold limit values.

## Section 3 – Physical/Chemical Characteristics

Boiling Point:		Not Applicable

Vapor Pressure:	Not Applicable

Melting Point:		120-170  C

Vapor Density:		Not Applicable

Evaporation Rate:	Not Applicable

Water Solubility: 	Not Applicable

Odor:			Essentially odorless

## Section 4 – Fire and Explosion Hazard Data

Flash Point:		>600  F

Extinguishing Media:	Dry Chemical, CO2, Foam, Water, Halon

### Special Fire Fighting Procedure

*Avoid inhalation of vapors

*Use self-contained breathing apparatus in confined areas

### Unusual Fire and Explosion Hazards

*Treat as a solid that can burn. Generally burns slowly with low smoke density and flaming drips. Burns with high smoke density under certain conditions.

## Section 5 – Reactivity Data

Material is stable. Hazardous polymerization will not occur.

## Section 6 – Health Hazard Data

#### Primary Routes of Entry

Inhalation – Negligible

Skin Contact – Negligible

Ingestion – Not applicable

*Not a carcinogen

#### Emergency and First Aid Procedure

Eye Contact – Flush with water

Skin Contact – Treat as thermal burn if contact with molten

## Section 7 – Precaution for Handling and Use

Practice reasonable care and caution in handling.

Waste Disposal: Place in appropriate disposal facility in compliance with state and federal regulations.

Storage: Store in a cool, dry location away from oxidizing materials.

## Section 8 – Control Measures

Use NIOSH respirators when hot molten product.

Protective Gloves: Required when Handling Molten Product.

Practice general hygiene by washing hands and clothes after handling.